Center for Education Projects
Education Improvement Project
ENVIRONMENTAL and Social MANAGEMENT PLAN FOR

rehabilitation of HIGH SCHOOL
in Ararat town of Ararat marz
 


Yerevan 2016

PART A: General Project and Site Information

	INSTITUTIONAL & ADMINISTRATIVE

	Country
	Republic of Armenia

	Project title 
	Education Improvement Project (EIP)

	Scope of site-specific activity
	Rehabilitation of a high school building in Ararat town of Ararat marz

	Institutional arrangements (WB)
	Task Team Leaders:

Nino Kutateladze
Soren Nellemann
	Safeguards Specialists:
Darejan Kapanadze (environment)

Sophia Georgieva (social)

	Implementation arrangements (RoA)
	Implementing entity
Center for Education Projects (CEP)
	Works supervisor
(tbd)
	Works contractor
(tbd)

	PROJECT AND SITE DESCRIPTION

	Project description
	Facilities of the high school located in Ararat town of Ararat marz are in poor physical condition. The school complex was built in 1980s and includes 5 buildings, 3 of which are two-storey, one three storey (including hall / large auditorium) and another one-storey building (gym). The investigation was carried out during design stage to reveal structural issues in the buildings. The following issues were observed in the buildings: 
· Metallic elements are corroded;

· The internal reinforcement bars of the columns are not anchored (welded) to the corner elements;

· The concrete of the external walls of the buildings is eroded and reinforcement bars and visible;
· Prefabricated monolithic concrete slabs have draw downs, as well as cracks;
· Load-bearing column in one of the buildings in absent, which impacts the structural integrity of the building and its seismic stability;

· The roof drainage is in poor condition and water can penetrate into the foundation and impact the bearing ability of clayey soil.

The following rehabilitation activities will be implemented under the proposed project: 

· Arrange entrance part, build a portal (faced with travertine tiles) with new entrance platform and ramp;

· Minimize heat loses by extending the walls between the windows and thermo-insulating the walls with mineral wool from outside, covering then with cement-sand plaster and then painting;

· Replacement of flooring;

· Replacement of wall plaster and paint;

· Replacement of partition walls;
· Replacement of doors and windows;

· Replacement of ceiling plaster, in some areas installation of hanging plastic ceilings.

Since the duration of the construction works is estimated to be 25 months, appropriate arrangements will be made for ensuring safe and convenient schooling process for students attending the school under rehabilitation. During the course of civil works the teaching process will be shifted to another school. The schooling will take place in the Ararat Basic School No 1. Ministry of Education and Science will manage the process through issuing the respective decrees. The empty building of the high school will be handed over to Construction Contractor to carry out the civil works (Contractors has no duties related to transferring). 

The final design for rehabilitation of the school in Ararat town of the RA Ararat marz was developed by the “BABAYAN-LAT NAKHAGITZ” LLC and “MODUL-2015” LLC.

	Name of School
	Ararat High School No 3.



	Address and site location of a school
	Address of the school: 67 Khanjyan str., Ararat town, Ararat marz.

Project site is located in Ararat town, Ararat marz, at a distance of about 50 km from the city of Yerevan. The site is situated in a developed urban area. All the works are envisaged to be implemented within existing school site.  



	Land use
	Land of school site and buildings on it are owned by the school / Ministry of Education and Science (certificate number 821119 dated 17.04.2002). Size of the land plot is 1.6 ha. The area of the school building is 4111.55 sq.m and the area of electric substation is 40.9 sq.m.
There are no other business activities carried out on the school’s land plot and buildings. In total 5 productive (2 apricot and 3 mulberry) and 16 non-productive trees are growing on the land plot. The trees will not be impacted during construction works.


	Description of physical and natural environment around the site; Social and demographic context
	The existing school to be rehabilitated is located in the Ararat marz of Armenia in the administrative area of Ararat community. The project area is located in Ararat valley characterized with dry continental climate. Elevation of the school site is about 830m above sea level. 
Summers are warm, average air temperature in July is 24-26°C. Maximum temperature in summer sometimes reaches up to 42°C and more. Winters are cold, average air temperature in January varies between -3°C to -5°C. Lowest observed temperature comprised -38°C. 

The main water arteries of the region are Araks, Hrazdan, Azat and Vedi rivers. The annual amount of precipitations is 200-300mm. Stable snow cover is not formed. Number of days with snow blanket is 39. The annual evaporability is above 1200mm. Air humidity is about 65%. The number of ten-day droughts is above 12.
Ararat town is situated at 47 km distance from Yerevan and it was founded in 1936. It is known as an industrial centre. Manufacturing is the main trend of industry of the town, in which manufacture of other non-metallic mineral products (manufacture of cement, lime carbonate, etc) comprises its prevailing part. 

Social outcomes of the Project are expected to be positive. The total number of permanent residents in the Ararat town is 20369 as of 1 January 2015 (including 48% men and 52% women). 

Currently there are 177 students enrolled in high school (including 80 boys and 97 girls), as well as 28 people are employed by the school (including 6 men and 22 women).



	Locations and distance for material sourcing, especially aggregates, water, stones?
	Existing school is connected to water supply and sewerage utilities. Water for construction works will be taken from the source agreed with the Head of Community and School principal (if the school utilities are proposed to be used). Aggregates will be obtained from the licensed providers within the project area. Contractor may choose to extract aggregates himself, in which case contractor must obtain an extraction license prior to commencement of extraction. All the materials will be purchased from an official supplier. No hazardous materials (asbestos-containing materials, lead containing paints, etc.) shall be used during rehabilitation works.

	LEGISLATION

	National & local legislation & permits that apply to project activity
	Environmental permits required for accomplishing the works envisaged by the project:

· Construction license to be possessed by Construction Contractor,
· Construction permit to be obtained by the Construction Contractor from municipality,
· Mining license to be possessed by Construction Contractor in case it operates a borrow pit,
· Agreement for disposal of construction waste to be obtained by Construction Contractor from municipality. 



	PUBLIC CONSULTATION

	When / where the public consultation process will take /took place
	Public consultation was carried out in the project community. Draft ESMP was discussed following its review by the World Bank, and the questions and concerns raised by the local beneficiaries were taken into account. Announcement on consultation was posted in the community before the consultation, and specifically parents, teachers, and other school employees were informed on it. Annex I presents the minutes of public consultation, and its appendices provide the list of participants and photographs.


	ATTACHEMENTS

	Site Map
	[image: image1.png]





	Copies of licenses and permits
	To be added once provided.



PART B: safeguards information

	ENVIRONMENTAL /SOCIAL SCREENING

	Will the site activity include/involve any of the following?
	Activity/Issue
	Status
	Triggered Actions

	
	A.  Building rehabilitation 
	[x] Yes  [ ] No
	See Section  A below

	
	B.  New construction
	[ ] Yes  [x] No
	See Section  A below

	
	C.  Individual wastewater treatment system
	[ ] Yes  [x] No
	See Section  B below

	
	D.  Historic building(s) and districts
	[ ] Yes  [x] No
	See Section  C below

	
	E.  Acquisition of land

	[ ] Yes  [x] No
	See Section  D below

	
	F. Hazardous or toxic materials

	[ ] Yes [x] No
	See Section  E below

	
	G. Impacts on forests and/or protected areas
	[ ] Yes  [x] No
	See Section  F below

	
	H. Handling / management of medical waste
	[ ] Yes  [x] No
	See Section  G below

	
	I. Traffic and Pedestrian Safety
	[ ] Yes  [x] No
	See Section  H below


PART C: Mitigation measures

	ACTIVITY
	PARAMETER
	MITIGATION MEASURES CHECKLIST

	0. General Conditions
	Notification and Worker Safety
	(a)  The local construction and environment inspectorates and communities have been notified of upcoming activities

(b)   The public has been notified of the works through appropriate notification in the media and/or at publicly accessible sites (including the site of the works)

(c)  All legally required permits have been acquired for construction and/or rehabilitation

(d)  The Contractor formally agrees that all work will be carried out in a safe and disciplined manner designed to minimize impacts on neighboring residents and environment.

(e)  Workers’ PPE will comply with international good practice (always hardhats, as needed masks and safety glasses, harnesses and safety boots)

(f)  Appropriate signposting of the sites will inform workers of key rules and regulations to follow.

	A. General Rehabilitation and /or Construction Activities
	Air Quality 
	(a) During interior demolition debris-chutes shall be used above the first floor

(b) Demolition debris shall be kept in controlled area and sprayed with water mist to reduce debris dust

(c) During pneumatic drilling/wall destruction dust shall be suppressed by ongoing water spraying and/or installing dust screen enclosures at site

(d) The surrounding environment (sidewalks, roads) shall be kept free of debris to minimize dust

(e) There will be no open burning of construction / waste material at the site

(f) There will be no excessive idling of construction vehicles at sites 

	
	Noise
	(a) Construction noise will be limited to restricted times agreed to in the permit

(b) During operations the engine covers of generators, air compressors and other powered mechanical equipment shall be closed, and equipment placed as far away from residential areas as possible

	
	Water Quality
	(a) The site will establish appropriate erosion and sediment control measures such as e.g. hay bales and / or silt fences to prevent sediment from moving off site and causing excessive turbidity in nearby streams and rivers.

	
	Waste management
	(a) Waste collection and disposal pathways and sites will be identified for all major waste types expected from demolition and construction activities.

(b) Mineral construction and demolition wastes will be separated from general refuse, organic, liquid and chemical wastes by on-site sorting and stored in appropriate containers.

(c) Construction waste will be collected and disposed properly by licensed collectors

(d) The records of waste disposal will be maintained as proof for proper management as designed.

(e) Whenever feasible the contractor will reuse and recycle appropriate and viable materials (except asbestos)

	B. Individual wastewater treatment system
	Water Quality
	(a) The approach to handling sanitary wastes and wastewater from building sites (installation or reconstruction) must be approved by the local authorities

(b) Before being discharged into receiving waters, effluents from individual wastewater systems must be treated in order to meet the minimal quality criteria set out by national  guidelines on effluent quality and wastewater treatment

(c) Monitoring of new wastewater systems (before/after) will be carried out

(d) Construction vehicles and machinery will be washed only in designated areas where runoff will not pollute natural surface water bodies.

	C. Historic building(s)
	Cultural Heritage
	(a) If the building is a designated historic structure, very close to such a structure, or located in a designated historic district, notification shall be made and approvals/permits be obtained from local authorities and all construction activities planned and carried out in line with local and national legislation.

(b) It shall be ensured that provisions are put in place so that artifacts or other possible “chance finds” encountered in excavation or construction are noted and registered, responsible officials contacted, and works activities delayed or modified to account for such finds.

	D. Acquisition of land
	Land Acquisition Plan/Framework
	(a) If expropriation of land was not expected but is required, or if loss of access to income of legal or illegal users of land was not expected but may occur, that the Bank’s Task Team Leader shall be immediately consulted.
(b) The approved Land Acquisition Plan/Framework (if required by the project) will be implemented

	E. Toxic Materials
	Asbestos management
	(a) If asbestos is located on the project site, it shall be marked clearly as hazardous material
(b) When possible the asbestos will be appropriately contained and sealed to minimize exposure

(c) The asbestos prior to removal (if removal is necessary) will be treated with a wetting agent to minimize asbestos dust

(d) Asbestos will be handled and disposed by skilled & experienced professionals

(e) If asbestos material is stored temporarily, the wastes should be securely enclosed inside closed containments and marked appropriately. Security measures will be taken against unauthorized removal from the site.

(f) The removed asbestos will not be reused

	
	Toxic / hazardous waste management
	(a) Temporarily storage on site of all hazardous or toxic substances will be in safe containers labeled with details of composition, properties and handling information 
(b) The containers of hazardous substances shall be placed in an leak-proof container to prevent spillage and leaching

(c) The wastes shall be transported by specially licensed carriers and disposed in a licensed facility.

(d) Paints with toxic ingredients or solvents or lead-based paints will not be used

	F. Affected forests, wetlands and/or protected areas
	Protection
	(a) All recognized natural habitats, wetlands and protected areas in the immediate vicinity of the activity will not be damaged or exploited, all staff will be strictly prohibited from hunting, foraging, logging or other damaging activities.
(b) A survey and an inventory shall be made of large trees in the vicinity of the construction activity, large trees shall be marked and cordoned off with fencing, their root system protected, and any damage to the trees avoided

(c) Adjacent wetlands and streams shall be protected from construction site run-off with appropriate erosion and sediment control feature to include by not limited to hay bales and silt fences

(d) There will be no unlicensed borrow pits, quarries or waste dumps in adjacent areas, especially not in protected areas.

	G. Disposal of medical waste
	Infrastructure for medical waste management
	(a) In compliance with national regulations the contractor will insure that newly constructed and/or rehabilitated health care facilities include sufficient infrastructure for medical waste handling and disposal; this includes and not limited to:
· Special facilities for segregated healthcare waste (including soiled instruments “sharps”, and human tissue or fluids) from other waste disposal; and

· Appropriate storage facilities for medical waste are in place; and

· If the activity includes facility-based treatment, appropriate disposal options are in place and operational

	H. Traffic and Pedestrian Safety
	Direct or indirect hazards to public traffic and pedestrians by construction 
activities
	(a)   In compliance with national regulations the contractor will insure that the construction site is properly secured and   construction related traffic regulated. This includes but is not limited to
· Signposting, warning signs, barriers and traffic diversions: site will be clearly visible and the public warned of all potential hazards

· Traffic management system and staff training, especially for site access and near-site heavy traffic. Provision of safe passages and crossings for pedestrians where construction traffic interferes.

· Adjustment of working hours to local traffic patterns, e.g. avoiding major transport activities during rush hours or times of livestock movement 

· Active traffic management by trained and visible staff at the site, if required for safe and convenient passage for the public.

· Ensuring safe and continuous access to office facilities, shops and residences during renovation activities, if the buildings stay open for the public.


PART D: Monitoring Plan

	Activity
	What

(Is the parameter to be monitored?)
	Where

(Is the parameter to be monitored?)
	How

(Is the parameter to be monitored?)
	When

(Define the frequency / or continuous?)
	Why

(Is the parameter being monitored?)
	Who

(Is responsible for monitoring?)

	CONSTRUCTION PHASE

	Mobilization of contractor
	The community and school administration has been notified of upcoming activities
	at school principal’s office
	discussion/ observation
	1 visit before construction commencement
	Ensure stakeholder awareness on the upcoming works
	CEP 

	
	All legally required permits have been acquired 
	CEP’s and construction contractor’s offices
	review of documents
	1 visit before construction commencement
	Ensure quality of works;

Prevent disruption of future activities due to lacking documentation
	

	
	PPE is provided to and used by workers
	at construction site
	inspection
	monthly visits during construction
	Prevent heath damage, trauma, and casualties among contractor’s personnel
	

	Generation of emissions and dust
	Construction site  sprinkled/watered as needed in the course of dust-generating works; 

No open burning of construction / waste material at the site;

No excessive idling of construction vehicles at site
	at construction site
	inspection
	monthly visits during construction
	Prevent air pollution and minimize nuisance to nearby residents
	CEP 

	Generation of noise 
	Construction noise limited to day-time hours
	at construction site
	inspection
	monthly visits during construction
	Minimize nuisance to nearby residents
	CEP 

	Generation of waste
	Locations for temporary storage of waste pre-identified and used accordingly; 
Construction waste regularly collected and disposed at the agreed site
	at construction site
	inspection
	monthly visits during construction
	Prevent littering of work site, pollution of soil and ground water
	CEP 

	Generation of hazardous waste (such as paints, solvents, etc.)
	Trained personnel using appropriate PPE is involved in dismantling, transportation and disposal works
	at school site,

at disposal site
	Inspection, review of documents
	regular visits
	Prevent contamination
	CEP,

Ministry of Nature Protection

	Nuisance to nearby residents caused by improper parking of construction machinery and vehicles, temporary storage of construction material and waste, or littering around the construction site by contractors
	No parking of construction vehicles and machinery outside the construction site;

No blocking of pedestrian and vehicle movement around the construction site due to stockpiling/dumping of construction materials/waste;

No trespassing of private land plots and/or other property around the construction site by contractor’s personnel.   
	in the immediate vicinity of the construction site
	inspection
	monthly visits, or if notified by contractor or citizens
	Prevent negative impacts on property, assets or livelihoods
	CEP 

	Works in a children’s institution
	Works carried out when school is out of operation; 
Appropriate warning signs installed and clearly visible to warn public of all potential hazards; 
In case that the school premises are used during the time of construction (for extra-curricular programs, by school staff, etc.),their safety is guaranteed and alternative arrangements made where necessary
	at construction site
	inspection
	monthly visits during construction
	Ensure safety of students and school employees
	CEP 

	Works within the settlement
	Warning signs are installed and clearly visible to warn public of all potential hazards;

Access to construction site is restricted to only authorized personnel involved in implementation of construction works, access to school site is strictly monitored;
Safe passages for pedestrians are provided
	at construction site

	inspection
	monthly visits during construction
	Ensure work site safety 


	CEP

	Grievance and redress mechanism (GRM)
	CEP and World Bank GRM information is available and visible to the public
	at locations for posting GRM information;

CEP office
	inspection
	monthly visits during construction
	Provide for all citizens in the community a channel to voice questions, feedback or complaints related to construction works
	CEP

	OPERATION PHASE

	School facility management
	School facilities are properly operated and maintained, including heating and ventilation, power supply, etc.
	at school site
	Inspection, review of documents
	regular visits
	Provide safe and convenient schooling environment
	Ministry of Education and Science

	Solid waste management
	Waste is regularly collected and transported from the school and disposed at the agreed site
	at school premises
	inspection, review of documents
	regular visits
	Maintain adequate sanitary conditions
	Ministry of Education and Science

	Water management
	Water supply and sewerage systems are properly maintained and are in good operational conditions; 
Drainage systems are properly maintained and are in good operational conditions
	at school premises
	inspection, review of documents
	regular visits
	Maintain adequate sanitary conditions
	Ministry of Education and Science

	Daily operation of the school
	Appropriate warning signs are installed and clearly visible to warn traffic on school located nearby, proper passages for pedestrian are arranged
	at school premises
	inspection, review of documents
	regular visits
	Provision of safe learning environment
	Ministry of Education and Science; 
Road Police

	Daily operation of the school
	First aid kit is available at school, teachers are trained on how to provide first aid and contact nearby medical station / hospital;

School area is properly fenced, handrails and stair are in good technical condition
	at school premises
	inspection
	regular visits
	Provision of safe learning environment
	Ministry of Education and Science

	
	Evacuation plans are posted on public areas in school and emergency exits are clearly marked, students and teachers are informed on activities to be undertaken in emergency situation;

School in equipped with appropriate fire-fighting means
	
	
	
	
	Ministry of Education and Science;

Ministry of Emergency Situations

	Grievance and redress mechanism (GRM)
	Ensure that GRM information is available and visible to the public
	at school site
	inspection
	regular visits
	to ensure that all citizens in the community have a channel to voice questions, feedback, or complaints related to the project
	[Ministry of Education and Science


ANNEX I. MINUTES, LIST OF PARTICIPANTS, PHOTOGRAPHS OF PUBLIC CONSULTATION
Minutes of Public Consultation Meeting on

Environmental and Social Management Plan developed for

rehabilitation of High School in Ararat town within the framework of Education Improvement Project

April 13, 2016







Ararat town

The meeting was summoned at 14:00.

In total, 45 participants attended the meeting, including representatives of the school staff, Center for Education Projects, beneficiary community. Participants of the public consultation have registered in the List of Participants and provided their contact details (Appendix 1). Photographs made during public consultation are presented in the Appendix 2. 

Mr. P. Qaramyan, representative of Center for Education Projects opened the meeting, presented the purpose of public consultation and briefly provided details on Education Improvement Project (EIP) preparation and implementation. Mr. Qaramyan emphasized the importance of environmental and social analyses conducted during the preparatory phase of the project, as well as preparation of designs. Mr. Qaramyan noted that the final version of the ESMP will be posted at the web-site and can be also requested from CEP in electronic and/or printed copy whenever needed. 

The announcement on public consultation was posted at the school door, in nearby shops, municipality before the consultation date. In addition, the representatives of school benefiting from Education Improvement Project were also invited by phone calls to attend the consultation. Mr. Qaramyan welcomed participants and briefly presented the key information regarding the EIP, including works planned under the various components of the proposed project.

Following opening remarks, environmental expert presented the Environmental and Social Management Plan developed for High School of Ararat town within the framework of Education Improvement Project. It was emphasized that the Environmental and Social management Plan was prepared in accordance with the requirements of the relevant RA legislation and World Bank’s Safeguard Policies. The potential environmental impacts associated with construction and rehabilitation works, as well as major mitigation measures that could be used to prevent or minimize the impacts, and monitoring measures were presented in details. 

Afterwards, the participants were provided with an opportunity to voice their feedback regarding the measures proposed in the Environmental and Social Management Plan, as well as raise their questions. Comprehensive answers were provided to all the questions.

The main questions raised during the consultation and responses provided are summarized below.

Question 1: Whether the landscaping / greening of the school area will be carried out?

Answer: The design envisaged landscaping / greening of the school area.

Question 2: Whether it is proposed to transfer the students to another school to carry out construction works, and if yes, in which school the teaching process will take place?

Answer: The issue is currently being discussed and in coming days the decision will be made regarding the school where the teaching will take place. Currently we are considering several options and envisaging that the students and teaching personnel will be transferred to one of nearby schools.

Question 3: How many years the construction works in the schools are expected to last?

Answer: According to the work schedule included in the design it is expected to carry out the construction works during 24 months.

Question 4: When the rehabilitation works are expected to commence?

Answer: The documentation on construction works is currently being prepared and upon finalization will be submitted to World Bank for approval, and after the approval the tender will be announced and then the construction works will commence.

Question 5: Whether it is proposed to strengthen the foundation of the school buildings to improve seismic stability?

Answer: The design envisages strengthening of foundations of school buildings and improvement of seismic stability.
Question 6: Which principles will be taken for implementation of supervision over construction works?

Answer: The supervision over implementation of construction works will be carried out by construction supervisor, as well as the author supervision of the design will take place, and the overall process will be supervised by the CEF of the Ministry of Education and Science.
The meeting was closed at 15:00.

The minutes were prepared by: 


High School Specialist of CEP



/signature/ N. Asryan
Director of High School of Ararat town:


/signature/ R. Nahapetyan

Appendix 1. List of participants of public consultation

[image: image2.emf]


F{
q)J41l-aF-t



q., g*
l<)
ad.l., 



A
E5Eoa
ch h*E



l\\)\\I
qq



rk\uPr
N



\
*\ [
Y



\!
\



q*J\\i\=*.J



dtd



J5q'-' 
.



d5,,
aA 



Fr
t-"J



c 
*{ 



aJ



dca-) 
t1



q, 
F{



,Fl 
AJ



!r/ 
A



&,8
UFI
(!=tr=o5
U3H



*)t'-



on\\o\hs



!,.1



^/V,>a0os/)
0r-
O



It(*1*rryh(va.i!,.\(/.ba,ba



Ntf(/)



\,t\l\dh\rlq



du-)71Frdl-a
tr&
.931ra 



rl
o-i 



)'
u, 



7)
o\|+{ 



Fa
v-



\.)ddoE-
o- 



F.(
r.JJ
E" "8
aCU)Ud
olar&o{



L



e,,i
J(\*)tF*ssr+5,-JrJ]J]{lvl$_l



{:oJ\



Iq$\"t
',\),rs



qrl
trEdF{
c7r-i cf{
Acl
v)iH*Cdd
ZJJ{



*



,{'t /
\-*l*-.}



$



"YT-
.*J.NJFt, 



l



r.-5 
1



&tFI



t\L\J^
^)il\"J



'\I)-l){*{+\'ir,(



'v\J



Z^I
Z\



\
\



\
-t-b



d-
\ot-{ON7r{



elI



)a*.{CDF{Ja\_.{\/{-,^T F'L{1J



LKt- 
J



YJqdl-( 
!



,-) \
L1PP



t\
1J



traH 
\-{



t-J 
F{



t-J>) 
4t



t+, 
)



,) -.(
H" 



-,
t-) 



A
JL{
o5L55rCt -(-)
):r)'< 



E{
*JM



J
\i 



-)
t/a



F- 
rL{



(-{)
- tt4
(c 



u<
-=5t'IJ



ZFItr<F
i-ZUt:l
HEYU26i
CrlX
*EV
=sB
-{JA
5Pr
r-< 



?
5Hf,2\-/ 



l'.1



;1 
z



!"{ 
ct^/
l_FZr-'



):JFr{
)3,{-,^I .-(-) 



\O-
5R14
tJ 



.r<
al 



I-
\.,L 



11\



$l\l











[image: image3.emf]


r-l
r,J)l-{F-i



qJ 
e-.r



l-{ 
)



atr
.L' 



A
EFf,o4
ch4=a



T\$ wa\$



\,}*\--
N



, \+\J
r \.J
\E-



?a)_{
J5Fl-d



aA 
Fr



F'?J



fttC
uF)
C.J 



r<



u 
7-r



UF{
c6=
dpo5
U3



F{
riy)



\f\t
(}(q\t
\rqI



q})
"-.) )
{)



btt)t



\"."1



\
I\f\*\)(



\tlat)



\_b
fi*)(,



[,*f
t-?{



C\-r
F+"
FF-



\*i N\nt. r^-
\'\



a'=S
,_)b=
tA\
AA>\=
)k\



s,
'''*-3
*\,\JI
\L-\)
fr.*I



6i€;
'tr3I



hF,,s+-
{}



"-*#
rye{\I''\ft{*



,\:/



q}r\
C'*--
sl--J



S-
.${fn\"b\fu)'\5



+-{
u-aFlHAL!-



trN
"93
t) 



Fa
oai 



Fa'
c7'



O
\.



n'{*<
dtr
oE"



oJ 
i-{



iJJodO
i5



33Ud
ol



Far+O
I



o-\
\\+*



- \\5
*.f$SrJJ3



(s..
d\)'-\{'"=.s-



s.)({Js



\-( 
t\..



-q-is*.
\t1
\lo:-!



N1,iJ



J\\J-t},-
\i-5-'Fr--



t*---,J-J-.5sJ



J"\b



JJ.LF."}-Jb
.'l



,Jl"t*J 1',
q'\"3



J\}



r-\b--
.b'\*.S-



J3q\)
JJJ



-S*F=ees



-.{
\r*>\)*"\



s\"t.*.J



\({*'-\5v



qrl
?1 



H



cd 
Pl



t+ic/
aiH*cr, d
z 



,,1
'{



=fu--"\
\--5
'*N$



csQ



'*\=d.tS*;5-\+*=.\+
q{



)
.-r$J3)



-.I-
"\.)



.J
.))J-t}



a.\d
.\FJJJ
>-c



i-.tt-a



H='r-'L_
:***Y'l*=-J



J\*J*4tt.'e 
--



J\.
\-<\ll..<',



q**



JIJ*-\-?



rJY=*)
-.:-.hj-



lJ\



-J,
-tJJ-tJ3E,tc-t\



-



>.J
\,J.J



k*''\r'
\ $"'



\,>\
\-.
\\}.stJ nlt*JH-\ \;,



Jtl



RQ
-



\-\,\
\-/ \)it



to=-.,5--



.*-)
J.Jt'."-l
p-J



_st-:)
,.-"t|



\*<,



J,J- s ' \"''
\.-r



\*i_



(S-
' 



F=-
q-5



(S



a;\



.Y\
*$ ''
\+bF"}



)'*13
:S{J
s



ZNz\
\-



*c
*



.-f_r,_



-\
I



r"h
{)



ry
s\



\.'. ri))-{
\o?E{



Oc{,-:-.' 
ta



. *-(
:, 



(a
,r' 



ar<
,ts 



""-t
i' 



-l
,.)



>/



sl\
5/t-{- \<t/J)ajE{



co?\<



zdJ^I
o" 



'q
,J



i* 
z 



i=
11 



F;l 
F{ 



?
\./ 



.* -
a 



i! i).
X Y p=
,6= i=
? H= H L .'
= 



"Li cn 
i-/



=H? rd
-tjA\Lr.



9E= 
6a-



U 
H 



Cq?
= 



= 
e:-



=ZC-Jc 
5l



x, 
l)



=) Z5!'-' 
Fr{



3)jf^I5i-))2?g.{



\oC{L



I



c{\l
"1











[image: image4.emf]


;t-{r-t
qrg
l-( 



J
td+JA
dld



:i 
c,.



ooB
cntr



N
W



NN
-



ffiC
.G
\



N
N$



nL_-{



J'-l
c).sl,-1a



ca



- 
rr"-



e-iC
U



c,
.l-r 



*
(J 



r-t
t71 



g-t
vFl



!J 
F--l



U
 



F't
rtr')
nrl



H



trtc-5.
U



3
r1h)FldH



\\
(\



I



\sr\nI



$'qs\q5



\\d)I



\\q
I



\t*I



\C
}



\(-'r'-\)



\S)be\\\*\S)t



hT,qT\\\̂\)



N
..-



\
t



\s$.t\\'t"\.b\"



b)
q



\\toP)-rt4t



oaSru



\t(\NV\
\\t\AN



\\



\(h.'
iJ\



Yr,)



Tba\
t



(|\B



trNct5f 
r+a



u) 
v)



o\C
tr



cE-t-<
j.JJ



tr-ji
=aU)U
€



C
1



;at<



ci



\\-\).\('l\Y{\\\J\\J\



\\\\)\,'\J.d\



\"1\$Ut



s\\a\\\}-
*$Nt\\N\J
\-&



\{F.)
\N



\-\^$\)+\\\
-.-\\
\\)\



,.. \!
\A.J\-t5--



Q
b



N
\\N\.r.)



t/$$.r
\--



Y
bl



nr)
\Is=$.\)s)\U



"\)N
-Nd,U\)



\)d'H



\
:Jr_)



'-.a_-



'$c*l
1-)r!



\S5



\xJ.J



N
J\s



\r=
,5-:f.s)



qr-*J
-*\"



(= 
FlL)



.t+-H
l



(n=,lFF,
Z 



,.1-<



\\*.\J1t'l$



A.TNJJ>Y\l



\
N



I



\\Jd



$N\a\JNq,\)$-\\}



s



Js\NN
-\$\\)\s\\\)Nt\5-a\)\)
-$o\



$\I
$$*'.N"J\)\)
t\*sJ$\
==t-*>\



J.)\s-\*e



\"S}S
R



*J



\$\)



dJtr$).\!')



t\.r\$*\LJ\Y



a")\\J



\\\"r{'$5*rJ\J
\



\Q
.."



\\\_s
',\\\\\o\,v -\.J
.\_\-J\-)



rJ*t\



\\\
J\a



\C
\. <\rJ\)rJ



\
\)$J



\
++-S



$
ZNz\



\
\



\
-$ \-q



A\b
\



$<)



ZF:A.t-.
FZr 1 



[-]
\J 



F-(
l:'l 



\-<



=4 A. 
ITI



\-/ 
7 rr



N
'U



:-z)
qc?
\- 



E-i



FH
9



=se t-- 
4'



asa



-D
-J



=c-s
f,2"
!1 



r:-:



;Z>-( 
o//F<\tzl_



\oN!!











[image: image5.emf]


;t-{r-t
qrg
l-( 



J
td+JA
dld



:i 
c,.



ooB
cntr



N
W



NN
-



ffiC
.G
\



N
N$



nL_-{



J'-l
c).sl,-1a



ca



- 
rr"-



e-iC
U



c,
.l-r 



*
(J 



r-t
t71 



g-t
vFl



!J 
F--l



U
 



F't
rtr')
nrl



H



trtc-5.
U



3
r1h)FldH



\\
(\



I



\sr\nI



$'qs\q5



\\d)I



\\q
I



\t*I



\C
}



\(-'r'-\)



\S)be\\\*\S)t



hT,qT\\\̂\)



N
..-



\
t



\s$.t\\'t"\.b\"



b)
q



\\toP)-rt4t



oaSru



\t(\NV\
\\t\AN



\\



\(h.'
iJ\



Yr,)



Tba\
t



(|\B



trNct5f 
r+a



u) 
v)



o\C
tr



cE-t-<
j.JJ



tr-ji
=aU)U
€



C
1



;at<



ci



\\-\).\('l\Y{\\\J\\J\



\\\\)\,'\J.d\



\"1\$Ut



s\\a\\\}-
*$Nt\\N\J
\-&



\{F.)
\N



\-\^$\)+\\\
-.-\\
\\)\



,.. \!
\A.J\-t5--



Q
b



N
\\N\.r.)



t/$$.r
\--



Y
bl



nr)
\Is=$.\)s)\U



"\)N
-Nd,U\)



\)d'H



\
:Jr_)



'-.a_-



'$c*l
1-)r!



\S5



\xJ.J



N
J\s



\r=
,5-:f.s)



qr-*J
-*\"



(= 
FlL)



.t+-H
l



(n=,lFF,
Z 



,.1-<



\\*.\J1t'l$



A.TNJJ>Y\l



\
N



I



\\Jd



$N\a\JNq,\)$-\\}



s



Js\NN
-\$\\)\s\\\)Nt\5-a\)\)
-$o\



$\I
$$*'.N"J\)\)
t\*sJ$\
==t-*>\



J.)\s-\*e



\"S}S
R



*J



\$\)



dJtr$).\!')



t\.r\$*\LJ\Y



a")\\J



\\\"r{'$5*rJ\J
\



\Q
.."



\\\_s
',\\\\\o\,v -\.J
.\_\-J\-)



rJ*t\



\\\
J\a



\C
\. <\rJ\)rJ



\
\)$J



\
++-S



$
ZNz\



\
\



\
-$ \-q



A\b
\



$<)



ZF:A.t-.
FZr 1 



[-]
\J 



F-(
l:'l 



\-<



=4 A. 
ITI



\-/ 
7 rr



N
'U



:-z)
qc?
\- 



E-i



FH
9



=se t-- 
4'



asa



-D
-J



=c-s
f,2"
!1 



r:-:



;Z>-( 
o//F<\tzl_



\oN!!











[image: image6.emf]


F.
v-JJAa-r



FA



a.r r
r<J
F{ 



t1
/ 



l-r
!., 



)
EFEol
6trA



e<
$



$N;\:\
\



\-J"){\S*..'



N\



\""sN.
N.Nw



.1L.r
J5q),-J



lA 
Fr



Fd
ri 



st
fi,c
.l-J 



F)
C.J '{



+;l.{ 
F)



\, 
a1



4
.lJ 



'F-i
(JF{
CE7
!J 



r<
tr=osU3



Aig,Jt-|l-{
H>)



-+1(-"{-Jo"



4?



\\(J.\.-.
.5t-+-
\)\5-$-{$\Js



*.-$$\|.-...
Jy



+xys\rt.J



a--.J
\)3



Y\
hj



sJt,



\-s\\=: \
-S,S
\())
t.r



-3-. *\,U



==:.]3
s.\sF.'
et* *'



i\
s$\. u\



"\
-X*s'
.$



' 
-J....i



r\V 
i\J



r:>J i.-
i^$
ts.-



\
.!\



{\..I-$x-N\
*$\{s$.N
N$,t\



\tl"



r{
,r_a-lH?1



tr&oH
.r.{ 



J
!J 



;.
u) ]r
O



\.
Q



{;<
ddor. F{ 



}'-.<
&J ,l
c,d
a533U7
ol



dg{
O



I



r(\
t'\9"cJO



I



&5ac



\"s
I



11-
J'tt



N
I



taLul



\)



\oq1t



aO
aI



o0t



Se)
p>e



\\
I



\bf-I



s,s
I



(\s?0



Nt,I



ar)I
hL,€



{



fir06b



,tfr)qs\_\
\J'.$"



)..{*
hL\.)



$\\\\t{-\tq\



nq
,)/\N--"
+,+\).{a
C,J



t"\\J)
a\l\



Erl
EE(BF{
t4 



5
i 



tr,
Cct
V)T
H
€



2EH
s$-)\)J-s*q\)



Jrt4$
Jl-Ds,tsJ$)



.I\)-t,SL



\"L.):c*J-t\)\)



r-JJJJ



-r*)JJ"r^5\"l(



-rV)J
"--*=>
. il



\r,
./



-_)
\r--".*



\rt\3.*"{*\}J..JkJ",'JI-Jd't.)\



lI



\\
d\Y\\$\$*Q\I-$N.



$r
w



ZNZ\
H



\
a



*t-
tc



\c
Fr



bo



tlt I-{
-1t-{h*,J)a*-{CD-{-)L<)L{)^{ ?*<){JJd)-lL.\
-)aL<{-l4tJ-.(F)A\-{>-)\{>);{-)i+{J{J-{-)tt*-{{)--{-\I4f)J)\P-J,*5;{-)d^I5-(J.luJ{



ZJp<t-(
i-2:
f 



1 
Fr'] 



ljrr
'\-/ 



.+



= =3
e uk
-r1z 



;



zHH T
,,\ 



Fl 
vlt 



*



-rjZ-\



3;, ? 
6



:..E-e
UtrC
=zc-o':r



l-tzI?t-



\oNl-



q











[image: image7.emf]


F.
v-JJAa-r



FA



a.r r
r<J
F{ 



t1
/ 



l-r
!., 



)
EFEol
6trA



e<
$



$N;\:\
\



\-J"){\S*..'



N\



\""sN.
N.Nw



.1L.r
J5q),-J



lA 
Fr



Fd
ri 



st
fi,c
.l-J 



F)
C.J '{



+;l.{ 
F)



\, 
a1



4
.lJ 



'F-i
(JF{
CE7
!J 



r<
tr=osU3



Aig,Jt-|l-{
H>)



-+1(-"{-Jo"



4?



\\(J.\.-.
.5t-+-
\)\5-$-{$\Js



*.-$$\|.-...
Jy



+xys\rt.J



a--.J
\)3



Y\
hj



sJt,



\-s\\=: \
-S,S
\())
t.r



-3-. *\,U



==:.]3
s.\sF.'
et* *'



i\
s$\. u\



"\
-X*s'
.$



' 
-J....i



r\V 
i\J



r:>J i.-
i^$
ts.-



\
.!\



{\..I-$x-N\
*$\{s$.N
N$,t\



\tl"



r{
,r_a-lH?1



tr&oH
.r.{ 



J
!J 



;.
u) ]r
O



\.
Q



{;<
ddor. F{ 



}'-.<
&J ,l
c,d
a533U7
ol



dg{
O



I



r(\
t'\9"cJO



I



&5ac



\"s
I



11-
J'tt



N
I



taLul



\)



\oq1t



aO
aI



o0t



Se)
p>e



\\
I



\bf-I



s,s
I



(\s?0



Nt,I



ar)I
hL,€



{



fir06b



,tfr)qs\_\
\J'.$"



)..{*
hL\.)



$\\\\t{-\tq\



nq
,)/\N--"
+,+\).{a
C,J



t"\\J)
a\l\



Erl
EE(BF{
t4 



5
i 



tr,
Cct
V)T
H
€



2EH
s$-)\)J-s*q\)



Jrt4$
Jl-Ds,tsJ$)



.I\)-t,SL



\"L.):c*J-t\)\)



r-JJJJ



-r*)JJ"r^5\"l(



-rV)J
"--*=>
. il



\r,
./



-_)
\r--".*



\rt\3.*"{*\}J..JkJ",'JI-Jd't.)\



lI



\\
d\Y\\$\$*Q\I-$N.



$r
w



ZNZ\
H



\
a



*t-
tc



\c
Fr



bo



tlt I-{
-1t-{h*,J)a*-{CD-{-)L<)L{)^{ ?*<){JJd)-lL.\
-)aL<{-l4tJ-.(F)A\-{>-)\{>);{-)i+{J{J-{-)tt*-{{)--{-\I4f)J)\P-J,*5;{-)d^I5-(J.luJ{



ZJp<t-(
i-2:
f 



1 
Fr'] 



ljrr
'\-/ 



.+



= =3
e uk
-r1z 



;



zHH T
,,\ 



Fl 
vlt 



*



-rjZ-\



3;, ? 
6



:..E-e
UtrC
=zc-o':r



l-tzI?t-



\oNl-



q











Appendix 2. Photographs made during public consultation 
[image: image8.jpg]


   [image: image9.jpg]



[image: image10.jpg]


    [image: image11.jpg]



[image: image12.jpg]


   [image: image13.jpg]



� Land acquisition includes displacement of people, impacts on livelihoods, encroachment on any private property, crops, trees,  impacts to buildings or assets that are either owned, transferred, rented or illegally used, for example as a dwelling or to operate  a business (kiosks, etc.).


�  Toxic / hazardous material includes but is not limited to asbestos, toxic paints, noxious solvents, removal of lead paint, etc.





5
PAGE  

